COURSE CODE & NAME:  ECE1408- EMEBEDDED SYSTEM

NAME: AADARSHIKA K
ROLL NO: 192565081

EX NO: 1

ARDUINO UNO-BASED SMART HOME SAFETY AND OCCUPANCY MONITORING SYSTEM USING GAS, PIR AND ULTRASONIC SENSORS

Aim

To design and simulate a smart home safety and occupancy monitoring system using Arduino UNO and suitable sensors to detect hazardous conditions and human presence.

2. Software and Hardware Requirements

Software:

- Arduino IDE

- Proteus Design Suite

Hardware / Simulation Components:

- Arduino UNO

- Gas sensor

- PIR motion sensor

- Ultrasonic sensor

- LED indicators

- Buzzer

- Virtual Terminal

- Connecting wires

## Program code:

## const int MQ2_PIN = A0;

## const int PIR_PIN = 2;

## const int TRIG_PIN = 8;

## const int ECHO_PIN = 7;

## const int LED_PIN = 10;

## const int BUZZER_PIN = 9;

## const float DISTANCE_LIMIT = 10.0;  // cm

## void setup() {

## Serial.begin(9600);

## pinMode(MQ2_PIN, INPUT);

## pinMode(PIR_PIN, INPUT);

## pinMode(TRIG_PIN, OUTPUT);

## pinMode(ECHO_PIN, INPUT);

## pinMode(LED_PIN, OUTPUT);

## pinMode(BUZZER_PIN, OUTPUT);

## digitalWrite(TRIG_PIN, LOW);

## Serial.println("GAS, MOTION AND DISTANCE MONITOR");

## Serial.println("System Started");

## Serial.println("-------------------------------");

## }

## float getDistance() {

## digitalWrite(TRIG_PIN, LOW);

## delayMicroseconds(2);

## digitalWrite(TRIG_PIN, HIGH);

## delayMicroseconds(10);

## digitalWrite(TRIG_PIN, LOW);

## unsigned long duration = pulseIn(ECHO_PIN, HIGH, 30000UL);

## if (duration == 0) {

## return -1.0;

## }

## return duration * 0.0343 / 2.0;

## }

## void loop() {

## // This Proteus MQ-2 model gives HIGH when gas is simulated

## int gasState = digitalRead(MQ2_PIN);

## int motionState = digitalRead(PIR_PIN);

## float distance = getDistance();

## bool gasDetected = (gasState == HIGH);

## bool motionDetected = (motionState == HIGH);

## bool nearbyObject =

## (distance >= 0 && distance <= DISTANCE_LIMIT);

## bool alarm = gasDetected || motionDetected || nearbyObject;

## Serial.print("Gas: ");

## Serial.println(gasDetected ? "DETECTED" : "SAFE");

## Serial.print("Motion: ");

## Serial.println(motionDetected ? "DETECTED" : "NOT DETECTED");

## Serial.print("Distance: ");

## if (distance < 0) {

## Serial.println("No echo");

## } else {

## Serial.print(distance, 1);

## Serial.println(" cm");

## }

## if (gasDetected) {

## Serial.println("WARNING: GAS DETECTED!");

## }

## if (motionDetected) {

## Serial.println("WARNING: MOTION DETECTED!");

## }

## if (nearbyObject) {

## Serial.println("WARNING: OBJECT TOO CLOSE!");

## }

## if (alarm) {

## digitalWrite(LED_PIN, HIGH);

## digitalWrite(BUZZER_PIN, HIGH);

## Serial.println("SYSTEM ALERT");

## } else {

## digitalWrite(LED_PIN, LOW);

## digitalWrite(BUZZER_PIN, LOW);

## Serial.println("SYSTEM SAFE");

## }

## Serial.println("-------------------------------");

## delay(1000);

## }

OUTPUT:

Result

Thus, the Smart Home Safety and Occupancy Monitoring System was designed and simulated using Arduino UNO and suitable sensors. The system monitors gas conditions, human movement, and nearby object distance, and provides alerts according to the programmed conditions