COURSE CODE & NAME:  ECE1408- EMEBEDDED SYSTEM

NAME: AADARSHIKA K
ROLL NO: 192565081

EX NO: 2

FORMAL DESIGN OF EMBEDDED CONTROLLER USING FSM – TRAFFIC LIGHT CONTROLLER

1. Aim

To design and simulate a traffic light controller using a Finite State Machine (FSM) to control traffic signals in a systematic sequence.

2. Software and Hardware Requirements

Software:

- Arduino IDE

- Proteus Design Suite

Hardware / Simulation Components:

- Arduino UNO

- Red LED indicators

- Yellow LED indicators

- Green LED indicators

- Resistors

- Virtual Terminal (if required)

- Connecting wires

3. Program code:

// ----------------------------------------------------

// Formal Design of Embedded Controller Using FSM

// Traffic Light Controller

// Arduino UNO + Proteus

// ----------------------------------------------------

enum TrafficState

{

STATE_NS_GREEN,

STATE_NS_YELLOW,

STATE_EW_GREEN,

STATE_EW_YELLOW

};

TrafficState currentState = STATE_NS_GREEN;

// North-South traffic signal

const int NS_RED_PIN    = 2;

const int NS_YELLOW_PIN = 3;

const int NS_GREEN_PIN  = 4;

// East-West traffic signal

const int EW_RED_PIN    = 5;

const int EW_YELLOW_PIN = 6;

const int EW_GREEN_PIN  = 7;

void setup()

{

pinMode(NS_RED_PIN, OUTPUT);

pinMode(NS_YELLOW_PIN, OUTPUT);

pinMode(NS_GREEN_PIN, OUTPUT);

pinMode(EW_RED_PIN, OUTPUT);

pinMode(EW_YELLOW_PIN, OUTPUT);

pinMode(EW_GREEN_PIN, OUTPUT)

Serial.begin(9600);

}

void allOff()

{

digitalWrite(NS_RED_PIN, LOW);

digitalWrite(NS_YELLOW_PIN, LOW);

digitalWrite(NS_GREEN_PIN, LOW);

digitalWrite(EW_RED_PIN, LOW);

digitalWrite(EW_YELLOW_PIN, LOW);

digitalWrite(EW_GREEN_PIN, LOW);

}

void enterState(TrafficState state)

{

allOff();

switch (state)

{

case STATE_NS_GREEN:

digitalWrite(NS_GREEN_PIN, HIGH);

digitalWrite(EW_RED_PIN, HIGH);

Serial.println("STATE S0: NS GREEN - EW RED");

break;

case STATE_NS_YELLOW:

digitalWrite(NS_YELLOW_PIN, HIGH);

digitalWrite(EW_RED_PIN, HIGH);

Serial.println("STATE S1: NS YELLOW - EW RED");

break;

case STATE_EW_GREEN:

digitalWrite(NS_RED_PIN, HIGH);

digitalWrite(EW_GREEN_PIN, HIGH);

Serial.println("STATE S2: NS RED - EW GREEN");

break;

case STATE_EW_YELLOW:

digitalWrite(NS_RED_PIN, HIGH);

digitalWrite(EW_YELLOW_PIN, HIGH);

Serial.println("STATE S3: NS RED - EW YELLOW");

break;

}

}

void loop()

{

switch (currentState)

{

case STATE_NS_GREEN:

enterState(STATE_NS_GREEN);

delay(10000);

currentState = STATE_NS_YELLOW;

break;

case STATE_NS_YELLOW:

enterState(STATE_NS_YELLOW);

delay(3000);

currentState = STATE_EW_GREEN;

break;

case STATE_EW_GREEN:

enterState(STATE_EW_GREEN);

delay(10000);

currentState = STATE_EW_YELLOW;

break;

case STATE_EW_YELLOW:

enterState(STATE_EW_YELLOW);

delay(3000);

currentState = STATE_NS_GREEN;

break;

}

}

4.OUTPUT:

5. Result

Thus, the traffic light controller was designed and simulated using the Finite State Machine (FSM) concept. The controller operates the red, yellow, and green traffic signals in a predefined sequence to support systematic traffic management.